# Contraints

| Command | Category | Typical Use |
| --- | --- | --- |
| fix | Single-point | Supports (fixed, pinned, roller) |
| equalDOF | MPC | Node coupling, symmetry |
| equalDOFLoc | MPC | Local‐axis coupling |
| rigidLink | MPC (rigid) | Offset/rigid connections |
| rigidDiaphragm | MPC (rigid) | Rigid floor/roof diaphragms |
| Handler: |  |  |
| Transformation | ConstraintHandler | Eliminates DOFs (default) |
| Lagrange | ConstraintHandler | Lagrange multipliers for complex linkages |
| Penalty | ConstraintHandler | Approximate constraints via high‐stiffness |

Below is a classification of OpenSees section types by how they receive their material definition (direct properties vs. material tags), what kind of materials they use (uniaxial vs. ND), and the kinds of structural models in which they’re typically employed.

Here’s a concise, Markdown-formatted overview of the main constraint commands in OpenSees, where and how they’re used in structural analysis, and which are most common.

## 1. Overview of Constraints in OpenSees

Constraints in OpenSees are used to impose relationships between nodal degrees of freedom (DOF). They fall into two broad categories:

- Single-Point (Support) Constraints
– Fix or release individual DOFs at a node.

- Multi-Point Constraints (MPCs)
– Impose algebraic relationships between DOFs at two or more nodes.
– Useful for coupling, rigid links, diaphragms, etc.

## 2. Single-Point Constraints: fix

Use fix to restrain (fix) or free (release) any combination of the 6 DOFs (UX, UY, UZ, RX, RY, RZ) at a node.

# Syntax: fix <nodeTag> <DX?> <DY?> <DZ?> <RX?> <RY?> <RZ?>

fix    1    1   1   1    0    0    0    ;# Node 1 fixed in X,Y,Z; free rotations

- Where used?
– Defining supports (pinned, fixed, roller).
– Gravity‐only boundary conditions.

- Most common use:
– Fixing base nodes in frame/seismic analysis.

## 3. Multi-Point Constraints (MPCs)

### 3.1 equalDOF

Couples one or more DOFs of a “slave” node to those of a “master” node:

# Syntax: equalDOF <slaveNode> <masterNode> <dof1?> <dof2?> ...

equalDOF   5    1    1    2    3    ;# Slave node 5 UX, UY, UZ = master node 1

- Where used?
– Linking nodes in meshed connections (e.g., façade‐to‐frame).
– Imposing symmetry or enforcing rigid body motion between nodes.

- Why most used?
– Simple, no extra DOFs introduced.
– Stable with the default TransformationConstraintHandler.

### 3.2 equalDOFLoc

Same as equalDOF but uses a local coordinate orientation at the slave:

# Syntax: equalDOFLoc <slaveNode> <masterNode> <dofList> <orientNodeI> <orientNodeJ>

equalDOFLoc 6   2    1 2    7    8   ;# Local X,Y of node6 tied to node2

- Where used?
– When coupling in a rotated/local system (e.g., skewed diaphragms, non-orthogonal grids).

### 3.3 rigidLink

Creates a rigid connection between a master node and a slave node for selected DOFs, introducing a new rigid body DOF at the master:

# Syntax: rigidLink <type> <slave> <master> <rFlag?> <iNode?> <jNode?> ...

rigidLink  beam  12   10   1    1      2      3

- Types:
– beam, bar, cable (depending on how stiffness is transferred).

- Where used?
– Modeling offset elements (e.g., beams with eccentricity).
– When the “slave” node should follow the master in a truly rigid sense.

### 3.4 rigidDiaphragm

Enforces that all nodes in a “diaphragm zone” move as a rigid plate about a reference master node.

# Syntax: rigidDiaphragm <dir> <masterNode> <eleTag1> <eleTag2> ...

rigidDiaphragm  Z   20   101  102  103  ;# Floor diaphragm rigid in Z‐normal plane

- Directions (dir):
– X, Y, Z (normal to the diaphragm).

- Where used?
– Floor or roof diaphragms in multi-story buildings.
– Ensuring in-plane stiffness far exceeds out-of-plane.

## 4. Constraint Handler Types

When you define constraints, OpenSees uses a ConstraintHandler to enforce them during analysis. The most common handlers are:

- TransformationConstraintHandler (default)
– Eliminates constrained DOFs, reducing system size; best for equalDOF, fix.

- LagrangeConstraintHandler
– Introduces Lagrange multipliers; use when transformation isn’t feasible (e.g., certain rigid links).

- PenaltyConstraintHandler
– Adds large‐stiffness “springs” to enforce constraints approximately; rarely used in structural analysis.

You select one via:

constraints Transformation

# or

constraints Lagrange  ;# if using rigidLink with coupled rotations

## 5. Which Are Most Used?

- fix – universal for supports.

- equalDOF – simple multi-point coupling.

- rigidDiaphragm – floor/roof rigid zones in building models.

Together, these cover the vast majority of real-world structural analysis setups in OpenSees.

### Quick Reference Table

| Command | Category | Typical Use |
| --- | --- | --- |
| fix | Single-point | Supports (fixed, pinned, roller) |
| equalDOF | MPC | Node coupling, symmetry |
| equalDOFLoc | MPC | Local‐axis coupling |
| rigidLink | MPC (rigid) | Offset/rigid connections |
| rigidDiaphragm | MPC (rigid) | Rigid floor/roof diaphragms |
| Handler: |  |  |
| Transformation | ConstraintHandler | Eliminates DOFs (default) |
| Lagrange | ConstraintHandler | Lagrange multipliers for complex linkages |
| Penalty | ConstraintHandler | Approximate constraints via high‐stiffness |

Feel free to ask for examples or deeper dives into any of these!

# Section

## 1. Sections with Direct Material Properties

These sections take numerical inputs (e.g. E, A, I) directly on the section command—no separate material object is referenced.

| Section Type | Inputs Required | Material Definition | Typical Applications |
| --- | --- | --- | --- |
| Elastic Section | E, A, I (and optional α) | Direct E, A, I | Geometric-nonlinear beams/columns |
| Wide Flange Section | d, tw, bf, tf, A, Iz, Iy, J | Direct W-section props | Steel W-shapes in frame analysis |
| Elastic Membrane Plate Section | E, ν, thickness, density | Direct shell props | Thin-shell / plate bending analysis |
| Bidirectional Section | E1, E2, G12, ν12, thickness | Direct orthotropic props | Composite shells, laminated plates |

## 2. Sections Referencing UniaxialMaterial Tags

These sections require one or more uniaxialMaterial objects (steel, concrete, elasto-plastic springs) defined separately.

| Section Type | How It Uses UniaxialMaterials | Typical Applications |
| --- | --- | --- |
| Fiber Section | Fibers (patches/layers) of concrete & steel with material tags | RC columns/beams under large inelastic demands |
| Uniaxial Section | Single uniaxialMaterial defines uncoupled moment–curvature | Plastic hinge springs, beamWithHinges |
| Plate Fiber Section | 2D fiber discretization of shell with uniaxialMaterials | Nonlinear shell/wall panels |
| RC Section | Specialized fiber section for concrete + rebar distributions | Reinforced-concrete beam/column pushover |
| Isolator2spring Section | Two uniaxialMaterials in parallel for bi-linear isolation | Seismic isolation bearings |

## 3. Sections Referencing NDMaterial Tags

These sections use ndmaterial objects that capture full multi-axial behavior.

| Section Type | How It Uses NDMaterial | Typical Applications |
| --- | --- | --- |
| NDFiber Section | Fibers each assigned an ndmaterial | 3D fiber beams/columns for anisotropic or multi-axial materials |
| Bidirectional Section (alt) | Can be built upon NDMaterials | Advanced composite shell elements |

## 4. Aggregator & Parallel Sections

These let you combine simpler sections into a composite.

| Section Type | How It Works | Applications |
| --- | --- | --- |
| Section Aggregator | Takes existing section‐tags and sums their stiffness | Combine RC core + steel jacket, or multi-material cables |
| Parallel Section | Runs multiple uniaxial sections in parallel | Multi-spring isolation devices or damper arrays |

## 5. Quick “Where and When” Guide

- Linear / Geometric Nonlinear Beams

  - Use: section Elastic or section WideFlange

  - Structure: Steel or concrete beams/columns under service loads.

- Plastic‐Hinge / Concentrated Nonlinearity

  - Use: section Uniaxial + beamWithHinges

  - Structure: Frame members in pushover studies.

- Distributed Inelastic Behavior (RC / Steel)

  - Use: section Fiber or section RC

  - Structure: Seismic–demand RC frames, fragility analysis.

- Shells & Plates

  - Use: section ElasticMembranePlate, section PlateFiber

  - Structure: Slabs, walls, shell roofs.

- Seismic Bearings / Isolators

  - Use: section Isolator2spring, section Parallel

  - Structure: Base isolation, energy-dissipating devices.

# Elements

Here’s a structured classification of OpenSees elements by their elastic vs. inelastic behavior, what kind of material/section input they require, and typical applications.

## 1. Elastic Elements

These behave linearly; stiffness is computed directly from material properties (E, G) and cross‐section geometry (A, I).

| Element | Input Type | Section/Material Tag | Inputs Required | Typical Use |
| --- | --- | --- | --- | --- |
| elasticBeamColumn | Direct material properties | – | E, A, I | Simple frame/gravity‐only analysis |
| elasticTimoshenkoBeamColumn | Direct material properties | – | E, G, A, I, shear‐distortion factors | Account for shear deformations in deep beams |
| elasticBeamColumnWarping | Direct material properties | – | E, G, A, Iyy, Izz, J, Iw | 3D beams with warping effects |
| truss | Direct material properties | – | E, A | Pin‐jointed axial members |
| corotationalTruss | Direct material properties | – | E, A | Large‐rotation truss analysis |
| twoNodeLink / zeroLength | Material‐based (uniaxialMaterial) | <matTag> | Material tag for a uniaxialMaterial defined separately | Springs, gaps, damping, custom link behavior |

## 2. Inelastic (Non-Elastic) Elements

These allow material or geometric nonlinearity. They require a Section object that in turn references one or more uniaxialMaterial tags.

| Element | Section Type | Section Tag | Material Tag(s) | Typical Use |
| --- | --- | --- | --- | --- |
| forceBeamColumn | Fiber or Elastic Section | <secTag> | Fiber: many uniaxialMaterial for steel/concrete; |  |
| Elastic: E, A, I | Nonlinear frame analysis under monotonic or cyclic loads |  |  |  |
| dispBeamColumn | Fiber or Elastic Section | <secTag> | Same as forceBeamColumn | Explicit Newmark‐type integration of beam–column |
| corotationalForceBeamColumn | Fiber or Elastic Section | <secTag> | Same as forceBeamColumn | Large‐rotation nonlinear analysis |
| beamWithHinges | Elastic Section + Hinges | <secTag> | Elastic E, A, I plus two hinge sections (uniaxialMaterial) | Concentrated plastic hinges at element ends |
| elasticSection (helper) | — | <secTag> | E, A, I… | Use when only geometric nonlinearity is needed |
| fiberSection (helper) | — | <secTag> | List of patch/fiber definitions each with a uniaxialMaterial tag | Distributed fiber‐based nonlinear sections |
| sectionAggregator | — | <secTag> | Aggregates multiple existing section tags | Combine different section models (e.g. core + cover) |

## 3. “Direct” Material Elements

Some elements don’t use a Section object but reference a single uniaxialMaterial directly:

| Element | Material Tag | Use Case |
| --- | --- | --- |
| zeroLength | <matTag> | Springs, gap‐elements, discrete nonlinear links |
| twoNodeLink | <matTag> | Advanced link behavior (e.g., shear panel, dashpot) |
| bearing2d / bearing1d | <matTag> | Elastomeric bearings, lead‐rubber, friction bearings |

## 4. When / Where to Use Which

- Linear analysis, quick checks

  - Use elastic elements (elasticBeamColumn, truss) with direct E, A, I.

- Geometric nonlinearity only

  - Use dispBeamColumn or forceBeamColumn with an elasticSection (direct E, A, I) so you don’t need fiber materials.

- Material nonlinearity

  - Define a fiberSection (steel + concrete fibers via uniaxialMaterial), then use forceBeamColumn or dispBeamColumn.

- Plastic hinge models

  - Use beamWithHinges: specify elastic section + two hinge sections (each defined by uniaxialMaterial tags).

- Rigid offsets or small links

  - Use zeroLength or twoNodeLink: reference a single uniaxialMaterial.

- Bearings & isolators

  - Use bearing1d, bearing2d, or custom link elements—again, single‐material tags.

### Quick Decision Flow

Is the behavior linear? ──► Yes: use elastic elements (direct E,A,I)

│

No

│

Material nonlinear? ──► Yes: define fiberSection → use forceBeamColumn / dispBeamColumn

│

No

│

Geometric only? ────► Yes: define elasticSection → use forceBeamColumn / dispBeamColumn

Feel free to ask if you’d like TCL snippets or deeper examples for any specific element or use case!

Below is a structured mapping showing:

- Which section types feed into which elements

- Which elements require a geometric transformation (and which type)

- Typical structural‐engineering contexts where you’d pick each combination

## A. Section → Element Mapping

| Section Type | Inputs / Materials | Elements That Require It | Use Case |
| --- | --- | --- | --- |
| Elastic Section | Direct: E, A, I | forceBeamColumn, dispBeamColumn, corotationalForceBeamColumn (when only geometric nonlinearity is of interest) | Geometric‐nonlinear steel frame under service loads |
| Wide Flange Section | Direct W‐shape props: d, bf, tf, tw, A, Iy, Iz, J | elasticBeamColumn (with geomTransf), or fiber–based inelastic beams when you want a “true” W‐section geometry | Preliminary sizing of steel beams |
| Uniaxial Section | Single uniaxialMaterial tag | beamWithHinges | Concentrated plastic‐hinge pushover of RC/steel frames |
| Fiber Section | patch + layer definitions referencing multiple uniaxialMaterial tags | forceBeamColumn, dispBeamColumn, corotationalForceBeamColumn | Detailed RC/seismic analysis with P–M interaction |
| NDFiber Section | Fiber discretization referencing ndMaterial tags | forceBeamColumn (3D), specialized 3D beam–column elements | Anisotropic or multi‐axial fiber models, advanced composites |
| Plate Fiber / Elastic Membrane | Patches of fibers or direct E, ν, thickness | Shell/plate elements (ShellMITC4, quad) | Nonlinear shell/wall panels, slab bending |
| RC Section | RC‐specific fiber layout (cover + core + rebars) | forceBeamColumn, dispBeamColumn | RC beam/column pushover & time‐history |
| Isolator2spring / Parallel | Two or more uniaxialMaterial tags in parallel | zeroLength, twoNodeLink | Base‐isolation bearings, energy‐dissipating devices |
| Section Aggregator | References existing section tags | Any beam–column element that can accept aggregated sections | Composite jackets, staged retrofit models |

## B. Geometric Transformations → Elements

Many beam–column elements require a coordinate transformation (a CrdTransf object) to map their “basic” stiffness (axial + flexure) into the global 2D/3D space. You create it with:

geomTransf <transfType> <transfTag> <args…>

| transfType | Description | Elements That Use It | When to Use |
| --- | --- | --- | --- |
| Linear | Small‐displacement; assumes element axis does not rotate under load | elasticBeamColumn, forceBeamColumn, dispBeamColumn | Service‐level, small‐drift gravity or wind analyses |
| PDelta | Accounts for P-Δ second‐order moments (geometry stiffening effect under axial load and lateral displacement) | Same set as Linear beams + columns | Seismic pushover or time‐history where axial load × lateral displacement coupling is non‐negligible |
| Corotational | Large‐rotation; updates the element axis as nodes move, but still assumes small strains | corotationalForceBeamColumn, elasticBeamColumnWarping | Large‐rotation steel frames, nonlinear dynamic analysis with significant member rotations |

TCL Examples

# 2D linear transform

geomTransf Linear  1

# 2D PDelta transform

geomTransf PDelta  2

# 3D corotational transform

geomTransf Corotational 3

# Then define an element using that transform:

element elasticBeamColumn 10  1 2  210000 0.02 8e-6  1

element forceBeamColumn   20  3 4  30  GaussLegendre 5  2

element corotationalForceBeamColumn 30  5 6  20  GaussLegendre 5  3

## C. Putting It All Together: Typical Workflow

- Define Materials

  - uniaxialMaterial Steel01 1 ...

  - ndMaterial ElasticIsotropic 2 ...

- Define Sections

  - Direct: section Elastic 1 E A I

  - Fiber:

  - section Fiber 5 {

  - patch rect 1 ...

  - layer straight 2 ...

  - }

- Create Geometric Transforms (for beams/columns)

- geomTransf Linear 1

- geomTransf PDelta 2

- geomTransf Corotational 3

- Define Elements

  - Elastic check:

  - element elasticBeamColumn 100  nI nJ E A I transfTag

  - Nonlinear static pushover:

  - element beamWithHinges 200  nI nJ secTag1 secTag2 L1 L2 transfTag

  - Inelastic dynamic:

  - element corotationalForceBeamColumn 300  nI nJ secTag  GaussLegendre 5 transfTag

- Run Analysis

  - Select constraints Transformation (default) or other handlers

  - Set system, numberer, integrator, algorithm…

  - analysis Static or analysis Transient

By matching section type → element → geomTransf type, you ensure your OpenSees model captures the right combination of material and geometric behavior for your structural‐engineering problem. Let me know if you’d like a full example TCL script for a specific case (e.g., a 3-story RC frame under seismic loading)!

Here is a structured classification of OpenSees elements according to:

- Elastic vs Inelastic

- Linear vs Nonlinear

- Uses: Section tag, Material tag, or Direct material properties

- Requires Geometric Transformation or not

- Typical Structural Use

### 🔧 LEGEND

| Label | Meaning |
| --- | --- |
| ✅ | Yes / Required |
| ❌ | No |
| ⚠️ | Optional / Conditional |

## 🔩 Beam-Column Elements

| Element Name | Elastic / Inelastic | Linear / Nonlinear | Input Type | Uses GeomTransf? | Typical Use Case |
| --- | --- | --- | --- | --- | --- |
| elasticBeamColumn | Elastic | Linear | Direct (E, A, I) | ✅ Yes | Simple beams, gravity-only models |
| elasticTimoshenkoBeamColumn | Elastic | Linear | Direct (E, G, A, I, shear factor) | ✅ Yes | Short beams, shear deformation |
| beamWithHinges | Inelastic | Nonlinear | Section tags at ends | ✅ Yes | Plastic hinge modeling |
| dispBeamColumn | Inelastic | Nonlinear | Section tag (any) | ✅ Yes | Explicit integration; walls, beams |
| forceBeamColumn | Inelastic | Nonlinear | Section tag (any) | ✅ Yes | Accurate general-purpose beam analysis |
| flexureShearBeamColumn | Inelastic | Nonlinear | Section tag (any) | ✅ Yes | P-M-V coupling (Shear+Flexure) |
| MVLEM, SFI_MVLEM | Inelastic | Nonlinear | Section tag + detailed layers | ✅ Yes | RC shear walls |

## 🧱 Solid / Continuum Elements

| Element Name | Elastic / Inelastic | Linear / Nonlinear | Input Type | Uses GeomTransf? | Typical Use Case |
| --- | --- | --- | --- | --- | --- |
| quad, tri31 | Elastic | Linear | Section tag (Elastic, Plate) | ❌ No | Plane stress/strain (2D structures) |
| shell, shellDKGQ, etc. | Elastic | Linear / Nonlinear | Section tag (PlateFiber) | ❌ No | Thin shell structures |
| brick, bbarBrick, SSPbrick | Elastic | Linear | NDMaterial or E/G/ν directly | ❌ No | 3D continuum models |
| SSPbrickUP, bbarBrickUP | Inelastic | Nonlinear | NDMaterial + u-p | ❌ No | Soil–structure interaction (SSI) |

## 🧷 Truss Elements

| Element Name | Elastic / Inelastic | Linear / Nonlinear | Input Type | Uses GeomTransf? | Use Case |
| --- | --- | --- | --- | --- | --- |
| truss | Elastic | Linear | Direct (E, A) | ❌ No | Axial members only |
| corotationalTruss | Elastic | Geometric Nonlinear | Direct (E, A) | ⚠️ Internally | Cables, large deflection trusses |

## 🧷 Zero-Length & Link Elements

| Element Name | Elastic / Inelastic | Linear / Nonlinear | Input Type | Uses GeomTransf? | Use Case |
| --- | --- | --- | --- | --- | --- |
| zeroLength | Inelastic | Nonlinear | uniaxialMaterial tag(s) | ❌ No | Springs, dampers, interfaces |
| zeroLengthSection | Inelastic | Nonlinear | Section tag (fiber/uniaxial) | ❌ No | Complex hinges in zero length |
| twoNodeLink | Inelastic | Nonlinear | uniaxialMaterial tag(s) | ❌ No | Shear panels, dashpots, dampers |

## 🧲 Bearing & Isolation Elements

| Element Name | Elastic / Inelastic | Linear / Nonlinear | Input Type | Uses GeomTransf? | Use Case |
| --- | --- | --- | --- | --- | --- |
| elastomericBearing, FPBearing | Inelastic | Nonlinear | uniaxialMaterial + friction | ❌ No | Base isolation, bridge bearings |
| flatSliderBearing | Inelastic | Nonlinear | uniaxialMaterial + frictionModel | ❌ No | Isolation systems |

## 🏗️ Joint Elements

| Element Name | Elastic / Inelastic | Linear / Nonlinear | Input Type | Uses GeomTransf? | Use Case |
| --- | --- | --- | --- | --- | --- |
| beamColumnJoint | Inelastic | Nonlinear | Material / section tags | ❌ No | Joint panel zone modeling |
| elasticTubularJoint | Elastic | Linear | Direct E, G, A, geometry | ❌ No | Tubular steel joints |

## 🌍 Contact & Cable Elements

| Element Name | Elastic / Inelastic | Linear / Nonlinear | Input Type | Uses GeomTransf? | Use Case |
| --- | --- | --- | --- | --- | --- |
| simpleContact2D/3D | Inelastic | Nonlinear | frictionModel, material | ❌ No | Sliding/frictional contact |
| catenaryCable | Elastic | Geometric Nonlinear | Direct (E, A) | ⚠️ Internally | Hanging cables, tension-only |

## 🔩 Summary Table

| Property | Examples |
| --- | --- |
| Elastic + Linear | elasticBeamColumn, truss, quad, brick, shellDKGQ |
| Elastic + Geometric Nonlinear | corotationalTruss, catenaryCable |
| Inelastic + Nonlinear | forceBeamColumn, zeroLength, mvlem, bearing, beamWithHinges, contact2D/3D |
| Use Section Tag | forceBeamColumn, dispBeamColumn, mvlem, shell, zeroLengthSection |
| Use Material Tag | zeroLength, twoNodeLink, brickUP, SSPquadUP |
| Use Direct Material Props | elasticBeamColumn, truss, quad, brick, catenaryCable |
| Requires geomTransf | All beam–column elements (except contact/truss/brick/zeroLength types) |

Let me know if you want a matrix format or interactive markdown table version for quick comparison and reference during modeling.

Here's a clear guide on which OpenSees elements and modeling techniques are most commonly used for different structural systems: Truss, Beam, Frame, and Steel vs RC (Reinforced Concrete) structures.

### 🏗️ 1. Truss Structures

Common Elements Used:

| Element | Use Case |
| --- | --- |
| truss | Linear axial member (E, A) |
| corotTruss | Geometrically nonlinear trusses |
| trussSection | Truss with section tag (fiber) |
| zeroLength / twoNodeLink | Modeling pin or hinge connections |

Materials:

- Direct: E, A

- Sometimes uniaxialMaterial for nonlinear tension/compression

Transformation:

- ❌ Not required (no bending = no geomTransf)

### 🧱 2. Beams

Common Elements Used:

| Element | Use Case |
| --- | --- |
| elasticBeamColumn | Simple elastic beam |
| dispBeamColumn | Inelastic RC/steel beam |
| forceBeamColumn | Preferred for nonlinear analysis |
| beamWithHinges | Hinge-based plasticity |
| zeroLengthSection | For hinge simulation at beam ends |

Materials:

- RC: Fiber sections using uniaxialMaterial for concrete + steel

- Steel: steel01, steel02, Hysteretic materials

- Use section tag for nonlinear beams

Transformation:

- ✅ Required: geomTransf Linear, PDelta, or Corotational

### 🏛️ 3. Frame Structures (2D & 3D)

Common Elements Used:

| Element | Use Case |
| --- | --- |
| forceBeamColumn | Most accurate + widely used |
| dispBeamColumn | Good for predefined integration points |
| elasticBeamColumn | For linear elastic models (gravity, etc.) |
| beamColumnJoint | Explicit joint modeling |
| zeroLengthSection | Plastic hinge zones |
| MVLEM, SFI_MVLEM | For RC walls in frame models |

Materials:

- Use fiber section with concrete + steel

- Use uniaxialMaterial like Concrete01, Steel01, Hysteretic

Transformation:

- ✅ Required for all beam-column elements

### 🧱 4. RC (Reinforced Concrete) Structures

Common Elements Used:

| Element | Use Case |
| --- | --- |
| forceBeamColumn + fiber | Most common and accurate for RC members |
| MVLEM / SFI_MVLEM | For RC walls (shear and flexure interaction) |
| dispBeamColumn | For simpler nonlinearity |
| zeroLength / zeroLengthSection | Connections, hinges |
| quad, shell | For slabs or wall panels |

Sections:

- Use fiber sections with:

  - Concrete01/02/04 for concrete

  - Steel01/02/04 for rebar

Transformation:

- ✅ Yes, use geomTransf for beam-column

### 🧱 5. Steel Structures

Common Elements Used:

| Element | Use Case |
| --- | --- |
| elasticBeamColumn | Linear frame analysis |
| forceBeamColumn | Nonlinear response (P-M interaction) |
| beamWithHinges | Plastic hinge zones |
| zeroLength or twoNodeLink | Braces, dampers, connections |

Sections:

- Can use:

  - Uniaxial sections for simplified models

  - Fiber sections with Steel01, Hysteretic, Bilinear for nonlinear

Transformation:

- ✅ Required for all beam elements

### 🔚 Summary

| Structure Type | Most Used Element(s) | Section / Material Used | Geometric Transformation |
| --- | --- | --- | --- |
| Truss | truss, corotTruss | Direct E, A or uniaxialMaterial | ❌ No |
| Beam | forceBeamColumn, dispBeamColumn | fiberSection + uniaxialMaterial | ✅ Yes |
| Frame | forceBeamColumn, beamWithHinges | fiberSection or uniaxialSection | ✅ Yes |
| RC | forceBeamColumn, MVLEM, zeroLength | fiberSection (concrete + steel materials) | ✅ Yes |
| Steel | elasticBeamColumn, forceBeamColumn | uniaxialSection, fiberSection (steel) | ✅ Yes |

# Time Series , Patterns, Load

Yes—you can pack any number of nodal and element loads into a single pattern. In OpenSeesPy, once you open a pattern, every subsequent load() or eleLoad() call goes into that pattern until you start a new one.

## 1. Multiple Loads in One Pattern

# 1) Define a Constant TimeSeries (for static loads)

timeSeries('Constant', tag=1, factor=1.0)

# 2) Open Pattern #1 (gravity + live)

pattern('Plain', tag=1, timeSeriesTag=1)

#    a) Nodal point load at node 2

load(2, 0.0, -20.0, 0.0)

#    b) Another nodal load at node 3

load(3, 10.0, 0.0,  0.0)

#    c) Uniformly distributed load on element 5

eleLoad('-ele', 5, '-type', 'beamUniform', 0.0, -5.0, 0.0)

#    d) A second eleLoad—say a point‐moment at node 4 on element 5

eleLoad('-ele', 5, '-type', 'beamPoint', 0.5, 0.0, 0.0,  1000.0)

#           ↑         ↑        ↑         ↑       ↑

#        tag spec   location  axial   transverse  moment

#                     ξ ∈ [0,1]   load     load    load about local z

All four loads live in pattern 1. When you run your analysis, pattern 1’s loads are scaled by the pattern’s TimeSeries.

## 2. Variable Distributed Load (VDL)

For a linearly varying load along a beam you use the beamVarying type. You specify the start and end intensities:

# Still inside your open pattern…

# eleLoad on element 5: w1 = –2 kN/m at ξ=0 (node I), w2 = –8 kN/m at ξ=1 (node J)

eleLoad('-ele', 5,

'-type', 'beamVarying',

-2.0,   # wI axial (usually 0)

-10.0,  # wI transverse

-8.0,   # wJ transverse

0.0     # wJ axial (if needed)

)

Here, the local‐2 direction load varies from –10 kN/m at the I-end to –8 kN/m at the J-end.

## 3. Discrete (Interior) Element Loads

To drop a concentrated load (force or moment) at some point along the element (ξ between 0 and 1), use beamPoint:

# eleLoad: point‐load P = –15 kN at midspan (ξ=0.5) in local 2-direction

eleLoad('-ele', 5,

'-type', 'beamPoint',

0.5,   # ξ location (0 at node I, 1 at node J)

0.0,   # axial P

-15.0,  # transverse P

0.0    # moment Mz, if any

)

You can mix as many beamPoint, beamUniform, and beamVarying calls as you like in one pattern.

## 4. Putting It All Together

- TimeSeries

  - Static → Constant

  - Dynamic → Linear, Path, Trig, etc.

- Pattern

  - Open one or many patterns, each tied to a TimeSeries.

- Loads

  - Nodal: load(nodeTag, Fx, Fy, Mz)

  - Uniform: eleLoad(..., '-type', 'beamUniform', wAxial, wTransverse, wTorsion)

  - Variable: eleLoad(..., '-type', 'beamVarying', wI_ax, wI_tr, wJ_tr, wJ_ax)

  - Point: eleLoad(..., '-type', 'beamPoint', ξ, P_axial, P_trans, Mz)

- Analysis Control

  - Define integrator (LoadControl or DisplacementControl) after all patterns & loads.

### Example Quick Snippet

# --- Setup ---

wipe(); model('basic', '-ndm',2,'-ndf',3)

node(1,0,0);node(2,5,0)

fix(1,1,1,1);fix(2,0,1,1)

# ... materials, sections, elements ...

# --- Loads ---

timeSeries('Constant', tag=1)

pattern('Plain', tag=1, timeSeriesTag=1)

load(2,0.0,-20.0,0.0)                            # nodal

eleLoad('-ele',5,'-type','beamUniform',0.0,-5.0,0.0)    # UDL

eleLoad('-ele',5,'-type','beamVarying',0.0,-8.0,-4.0,0.0) # VDL

eleLoad('-ele',5,'-type','beamPoint',0.3,0.0,-10.0,0.0)  # discrete

# --- Analysis ---

constraints('Transformation')

integrator('LoadControl', incr=0.1)

analysis('Static'); analyze(10)

With this pattern‐based approach, you can flexibly build multiple load cases—each with any combination of point, uniform, varying, or discrete element loads—all in a single structural model.

You’ve got the right idea of the static vs. dynamic load workflow. Here’s how it all hangs together, and what (if anything) changes when you switch to a displacement‐controlled analysis.

## 1. Always: Define TimeSeries → Pattern → Loads

- TimeSeries

  - Static analysis: use a Constant TimeSeries (λ(t) ≡ 1).

  - Dynamic / time-history: pick a TimeSeries that describes how your loads vary in “analysis time” (Linear ramp, Trig, Path from a file, Ground-motion, etc.).

- LoadPattern

  - Each pattern groups together one TimeSeries and a set of loads.

  - You can—and often do—create multiple patterns: e.g.

    - Gravity pattern (self-weight) using Constant TS

    - Live-load pattern using Linear or Path TS

    - Seismic pattern using a Ground-Motion TS

- Apply Loads

  - Inside each LoadPattern you specify:

    - Nodal loads (point forces, moments)

    - Element loads (UDL, VDL, or discrete)

    - Single-point constraints (e.g. a controlled displacement in a multi-pattern analysis)

Regardless of how you drive the model (by load or displacement), you always need to define your loads in this TimeSeries→Pattern→Load sequence.

## 2. LoadControl vs. DisplacementControl

### A. LoadControl (Default for Static)

- You tell OpenSees:

  - “Increment the overall load factor λ by Δλ each step.”

  - At each step, each active pattern multiplies its defined load by the current λ.

- You see conventional P–Δ curves, pushover curves, etc.

### B. DisplacementControl

- Instead of prescribing Δλ, you prescribe a target displacement at a particular monitor DOF.

- Internally, OpenSees still uses the same TimeSeries→Pattern→Load definitions.

- But at each step it solves for the load factor λ that makes your monitored node/DOF reach the specified displacement increment Δu.

#### Do you still need the previous steps?

- Yes—you still must define TimeSeries, Patterns, and Loads exactly as before.

- The only thing that changes is the integrator/analysis control:

  - LoadControl Δλ → increments in load

  - DisplacementControl Δu on (node, dof) → increments in displacement

## 3. Why DisplacementControl Doesn’t Replace Load Definitions

- DisplacementControl tells how the nonlinear solver marches along the P–Δ path, but it does not replace what the loads are or when they’re applied.

- You may choose to ramp your “Live-Load” pattern with a Linear TimeSeries, but still control the analysis by displacement so that you can capture post-yield behavior or snap-through.

## 4. Typical Workflow for DisplacementControl

- Define all Loads, in one or more Patterns, each tied to a (usually Constant) TimeSeries.

- Activate the relevant Patterns (all of them, or selected ones) in the analysis.

- Choose DisplacementControl integrator and specify:

  - The monitor node and DOF

  - Target incremental displacement per step (Δu)

  - Maximum number of iterations or steps

- Run the static analysis. At each step, OpenSees will:

  - Scale the combined defined loads by the necessary λ so that the monitored displacement increases by Δu

  - Solve equilibrium under that λ × loads

### 🔑 Take-Home Message

- Loads are always defined by TimeSeries → LoadPattern → Load commands.

- Control scheme (LoadControl vs. DisplacementControl vs. ArcLength) only tells the solver how to march along the equilibrium path—it does not alter the definitions of your loads.

- You do not skip timeSeries or pattern definitions when you switch to DisplacementControl. Those remain mandatory.

# Analysis Integrator

In a static OpenSees (or OpenSeesPy) analysis, the integrator you choose tells the solver how to march along the equilibrium path. Two of the most common are LoadControl and DisplacementControl. They both work with any combination of patterns and loads you’ve defined—you just set one integrator or the other before you call analyze(). Here’s how each works, what they do under the hood, and what pitfalls to watch for.

## 1. LoadControl Integrator

### How it works

- Global “load factor” λ: All active load patterns (gravity, live load, seismic, etc.) are multiplied by the same scalar λ.

- Increment Δλ: You specify a constant increment, e.g. 0.1.

- At each step:

  - The solver applies all defined loads scaled by the current λ.

  - It assembles the global stiffness matrix K(𝑢) (which may depend on the current displacement 𝑢 if you have material or geometric nonlinearity).

  - Solves: K(𝑢) Δ𝑢 = Δλ·P, where P is the vector of full (unscaled) loads.

  - Updates displacements 𝑢 ← 𝑢 + Δ𝑢, increments λ ← λ + Δλ.

### Pros

- Intuitive: You ramp loads up in known, fixed steps.

- Straightforward: Works well if the structure remains globally stable under monotonic loading.

### Cons / Pitfalls

- Limit points: If the structure softens (snap-through or snap-back) so that the slope dP/d𝛿 → 0 or negative, the stiffness matrix can become singular or indefinite. The solver may fail to converge before you reach your target λ.

- Load jumps: Too large Δλ can cause poor convergence or skip over critical nonlinear behavior.

- No direct control of critical displacements—if you care about a particular drift or hinge rotation, LoadControl may overshoot or never reach it if λ stalls.

## 2. DisplacementControl Integrator

### How it works

- Monitor DOF: You pick one node and one degree of freedom (e.g., vertical displacement at story-5, DOF 2).

- Target increment Δ𝑢: You prescribe how much that DOF should move each step (e.g., 0.01 m).

- At each step:

  - The solver adjusts the global load factor λ so that when equilibrium is found, the monitored DOF has moved exactly by Δ𝑢.

  - In practice it solves a coupled system for both Δ𝑢 and Δλ to satisfy equilibrium and the displacement target.

### Pros

- Follows post-peak behavior: Even if the global load–displacement curve softens or snaps back, DisplacementControl will continue stepping along the displacement axis.

- Direct drift or rotation targets: You can capture the response at a critical location (e.g., roof drift, plastic hinge rotation).

### Cons / Pitfalls

- Choice of monitor DOF: If you pick a DOF that never moves (e.g., a fixed support), the solver can’t satisfy your target and will fail.

- Multiple critical DOFs: Structures with many hinges or complex kinematics may need multi-point or arc-length control—single DOF control can miss other instabilities.

- Load factor λ not directly known until you solve; you often have to query it afterward if you need the actual applied load.

## 3. Use with Multiple Patterns & Load Combinations

- Both integrators act on all active patterns simultaneously. If you have, say, a gravity pattern plus a live-load pattern plus a wind pattern, the current λ multiplies every load in every pattern.

- You can turn patterns on/off between stages (e.g., hold gravity constant while ramping live load), but within a single analyze() call each integrator simply scales all currently active patterns.

## 4. Practical Recommendations

- Start with LoadControl for your initial, pre-yield “service-level” checks—keep Δλ small enough to ensure convergence.

- Switch to DisplacementControl when you approach yielding or expect softening, so you can capture the full P–Δ path through peak and post-peak.

- Watch convergence:

  - If Newton doesn’t converge within your tolerance/iterations, reduce Δλ or Δ𝑢.

  - Tighten or relax your test tolerance as needed.

- Consider Arc-Length if you have snap-back behavior or multiple simultaneous instabilities that neither single-load nor single-displacement control can handle cleanly.

### Summary Table

| Feature | LoadControl | DisplacementControl |
| --- | --- | --- |
| Drives | Load factor λ | Displacement at (node, DOF) |
| Increment | Δλ (user-defined) | Δ𝑢 (user-defined) |
| Handles softening? | No—can fail at limit points | Yes—continues along displacement |
| Requires | Any load patterns | Any load patterns + valid DOF |
| Common use | Pre-yield, monotonic loading | Post-yield, snap-through |

Both integrators will happily work with any combination of patterns and loads you’ve set up—just choose the one that best traces the part of the response you care about.